87-90 Mustang Turbo Swap Instructions 
Thanks go to GT350R, SVO1, & Crovax - Authored by 140cilx, Formatted by Stinger

1. Intro

This is specific to the '87-90 LX cars receiving the '87-88 TC computer and harness.  

These notes are for a 5speed car only, but should be close if you wanted to do an auto trans conversion. (who would do that?)

First off try to use as many pieces as possible from an 87-88 Turbo Coupe, as it will simplify the conversion process. You do not need the whole wiring harness from the t-bird, only the VAM harness connector.  DO NOT remove the harness from the Mustang. 

Read this article carefully and you should be set on how to wire the conversion. if you do not understand something feel free to study that area until you do understand. If you do not you could end up messing up something that could be costly to repair.

Also feel free to email me with any questions, or if you find any errors in this article. I will not accept responsibility for anything you might fry in the process of doing this swap. I advise researching all the wiring diagrams and EEC IV info you can get your hands on.


I am not going to explain how to remove and install pins from a connector, if you can't figure it out you might not want to do this swap.  I will give you a hint though, a small screwdriver or small razor knife works well.


2. How to Connect VAM Wiring

From the VAM wiring harness from the Turbocoupe:

Connect the VRef (orange/white) to the wire going to pin 26 of the stock harness (also orange/white). (When I say connect I mean cut the connection, disregard the former wire running into the engine bay, and connect the new wire. This wire going to pin 26 can be found at your EGR plug. This means you will no longer use your EGR.)

Connect the VAF (white/black) to the wire going to pin 27 of the stock harness (brown/light green).  (This wire going to pin 27 can be found at your EGR plug. This means you will no longer use your EGR.)

Note: Remove unused Pin Connector #3 from LX wire harness red block (or take one from your TC harness you salvaged, and put this pin connector and wire into the red block of your LX harness to plug into your Pin 43 position on the TC computer. The LX harness lacks this connecting pin). Connect the Vane Air Temp (light green/purple) to Pin 43 using this transferred wire you have moved into position 43.

Note II: Splice into/"T" connect the VAM signal return (black/white) wire into the LX harness (black/white) wire that goes to pin 46. Do not cut the LX harness (black/white) wire going into pin 46, simply connect this VAM signal return black/white wire to it.


3. ACT Wire Conversion 

no changes necessary.
Connect ACT wire to the wire going to pin 25 (green/red). This is a plug in on the ’87-90 LX cars using the ’87-88 TC computer since we have an ACT sensor.


4. Premium Mode Wiring

Cut the wire going into pin #24 of the LX harness about 5” above the wiring block. This is an Air Conditioning Wire. The Air Conditioning will still work properly with this wire cut and capped – this was an idle increase control wire for the Air Cond.. The 5” section of wire remaining, going into the computer is your #24 Octane control. Take this wire and “T” this into pin #46 signal return. This will complete the connection and allow your computer to run in the “High Octane” setting. You must from this point on, run 91 or greater octane fuel. 


5. Closing

Unplug the vacuum hose going to your MAP sensor and leave it unplugged on the MAP, Then cap the vacuum tree port where the vacuum line plugged into the tree.

The above information does not include wiring in the Electronic (computer) Boost Control, so you will need to add a preferred (for greater boost) Adjustable Boost Valve, ignoring computer control of boost.  I prefer the Gillis Valve found at www.boostvalve.com.